UNIVERSITY OF BUEA

FACULTY OF ENGINEERING
AND TECHNOLOGY

DEPARTMENT OF
COMPUTER ENGINEERING

A DISTRIBUTED BLOCKCHAIN-BASED APPROACH TO
PROTECT IDENTITY, INTEGRITY AND PRIVACY IN
AGRICULTURAL DATA PROCESSING

By

FORCHA GLEN BELOA
(FE22P049)
M.Eng. in Telecommunications and Networks

A Thesis submitted to the Faculty of Engineering and Technology of the
University of Buea in partial fulfilment of the requirements
for the degree of Doctor of Philosophy (PhD)
in Computer Engineering.

Supervisors

Michael Ekonde SONE, PhD
Associate Professor

Elie FUTE TAGNE, PhD
Associate Professor

UNIVERSITY OF BUEA

FACULTY OF ENGINEERING
AND TECHNOLOGY

DEPARTMENT OF
COMPUTER ENGINEERING

A DISTRIBUTED BLOCKCHAIN-BASED APPROACH TO
PROTECT IDENTITY, INTEGRITY AND PRIVACY IN
AGRICULTURAL DATA PROCESSING

By

FORCHA GLEN BELOA
(FE22P049)
M.Eng. in Telecommunications and Networks

A Thesis submitted to the Faculty of Engineering and Technology of the
University of Buea in partial fulfilment of the requirements
for the degree of Doctor of Philosophy (PhD)
in Computer Engineering.

Supervisors

Michael Ekonde SONE, PhD
Associate Professor

Elie FUTE TAGNE, PhD
Associate Professor

## DEDICATION

To …

## CERTIFICATION

UNIVERSITY OF BUEA
FACULTY OF ENGINEERING AND TECHNOLOGY

This is to certify that the work described in the thesis entitled “A Distributed Blockchain-Based Approach to Protect Identity, Integrity and Privacy in Agricultural Data Processing”

By

Was carried out in the Department of Computer Engineering

Under the supervision of

(Name and Signature of Supervisor)

Date

## ACKNOWLEDGEMENTS

## ABSTRACT

## TABLE OF CONTENTS

## LIST OF FIGURES

## LIST OF TABLES

## LIST OF ABBREVIATIONS

ABAC

Attribute-Based Access Control

ACK

Acknowledgement

ADC

Analogue-to-Digital Converter

API

Application Programming Interface

ARM64

64-bit Advanced RISC Machine architecture

BATMAN-adv

Better Approach To Mobile Ad-hoc Networking, advanced

BLE

Bluetooth Low Energy

BFT

Byzantine Fault Tolerance

CA

Certificate Authority

CAP

Consistency, Availability, Partition tolerance

CPU

Central Processing Unit

CRT

Chinese Remainder Theorem

DID

Decentralised Identifier

DLT

Distributed Ledger Technology

DP

Differential Privacy

EDP

Energy-Delay Product

ESP32

Espressif 32-bit microcontroller with integrated Wi-Fi and Bluetooth

GDPR

General Data Protection Regulation

GW

Gateway

HLF

Hyperledger Fabric

HRBAC

Hierarchical Role-Based Access Control

IoT

Internet of Things

IPFS

InterPlanetary File System

JSON

JavaScript Object Notation

LoRa

Long Range (sub-GHz spread-spectrum radio modulation)

LoRaWAN

Long Range Wide Area Network

LZW

Lempel-Ziv-Welch compression

MAC

Medium Access Control

MCU

Microcontroller Unit

MSP

Membership Service Provider

PBFT

Practical Byzantine Fault Tolerance

PKI

Public Key Infrastructure

RAM

Random Access Memory

RBAC

Role-Based Access Control

RF

Radio Frequency

ROI

Return on Investment

RPi

Raspberry Pi

RTC

Real-Time Clock

SF

Spreading Factor

SHA

Secure Hash Algorithm

SLO

Service Level Objective

SSS

Shamir Secret Sharing

TEE

Trusted Execution Environment

TPS

Transactions Per Second

TPM

Trusted Platform Module

ZKP

Zero-Knowledge Proof

# 1	Introduction

## 1.1	Background to the Study

Agriculture is undergoing a transition from experience-driven practice to data-driven decision making. Networks of low-cost sensors now measure soil moisture, canopy temperature, humidity and nutrient status continuously, and the resulting streams feed irrigation controllers, yield models and certification schemes (Rathi and Gomathy 2025; Pereira, Oweis, and Zairi 2002). In water-stressed regions this shift is not a convenience but a necessity. Irrigation accounts for the dominant share of freshwater withdrawal, and climate variability has compressed the margin for error in scheduling decisions (Dono, Giraldo, and Severini 2010; Abioye et al. 2020).

A measurement taken in a field does not stop at the field. It becomes a record, and that record is consumed by parties with interests of their own, as Figure 1.1 sets out. The value of the record to those parties is exactly what creates the incentive to falsify it, and the further a record travels from the instrument that produced it, the weaker the evidence that it is genuine.

Lifecycle of an agricultural measurement. A reading becomes a record that is consumed by parties outside the farm, and the value those parties attach to it is what makes falsification worthwhile.

The economics of the sensing layer explain why the problem is hard rather than merely tedious. A deployment that is worth installing on a smallholding must cost a small fraction of the crop value it protects, which forces the node bill of materials down to a level at which neither a hardware security module nor a generous energy budget is affordable. The same economics rule out mains power and wired backhaul across a field, so the design space collapses onto battery-powered microcontrollers speaking a long-range, low-rate radio. Every protection mechanism proposed in this thesis has to survive inside that envelope, and a mechanism that works only on a mains-powered gateway has solved the easier half of the problem.

The sensing layer of such systems is built from severely constrained devices. An Espressif 32-bit microcontroller with integrated Wi-Fi and Bluetooth (ESP32) node operating on battery and harvested solar power must transmit over kilometre-scale distances using Long Range (sub-GHz spread-spectrum radio modulation) (LoRa), a modulation scheme that trades data rate for range and therefore charges a high airtime cost per packet (Haxhibeqiri et al. 2018; Adelantado et al. 2017). Airtime is the binding constraint. It determines energy consumption at the node, and, through duty-cycle regulation and collision probability, it determines how many nodes a single gateway can serve (Vukovic, Raca, and Vukovic 2023). Conventional responses compress the payload, but general-purpose compressors such as Huffman and Lempel-Ziv-Welch compression (LZW) carry dictionary and buffer overheads that a microcontroller with a few kilobytes of free Random Access Memory (RAM) can ill afford (Marcelloni and Vecchio 2008, 2010).

Two further constraints sharpen this. Sub-GHz Internet of Things (IoT) bands are governed by a duty-cycle limit, so a node that has just transmitted must stay silent for a period proportional to the airtime it consumed. Airtime is therefore not only an energy cost but a rate limit: it caps how often a node may report, and no increase in battery capacity relaxes it. Second, because LoRa uses unslotted access, packets collide whenever two nodes transmit on the same channel at the same time, and the collision probability grows with the square of offered traffic in the regime that matters. Halving per-packet occupancy therefore buys more than a proportional reduction in collisions, which is why the mechanism studied here is worth the arithmetic it costs.

The Chinese Remainder Theorem (CRT) offers a different route. Decomposing a reading into residues modulo pairwise coprime moduli yields several short packets in place of one long one (Garner 1959; Ding, Pei, and Salomaa 1996). Because a multi-channel concentrator such as the SX1302 demodulates channels concurrently, the short residue packets can be received simultaneously, and the original value is recovered in Ok operations for k moduli. The practical effect is a reduction in per-packet occupancy rather than in total transmitted volume, which is precisely the quantity that governs gateway capacity. The detailed encoding and reconstruction path is presented in Chapter Three.

Residue arithmetic is not new to distributed systems. The same decomposition has been applied to the throughput problem in permissionless blockchains, where splitting work across residue classes permits parallel processing of operations that would otherwise serialise. That line of work establishes the arithmetic and the parallelisation argument; what it does not address is the constrained radio link, where the payoff is airtime rather than processor cycles, nor the identity and confidentiality questions that arise when the residues cross a shared medium. This thesis takes the arithmetic as established and asks what it is worth on the edge of the network rather than at its centre.

Data that has been collected cheaply must then be trusted. Agricultural records increasingly carry consequences beyond the farm gate. They substantiate organic and fair-trade certification, they underwrite index-based crop insurance, and they inform subsidy allocation. Each of these uses creates an incentive to falsify. A centralised database concentrates that risk in a single administrator, and an unprotected sensor network invites the injection of fabricated readings by any party with a compatible radio. Distributed Ledger Technology (DLT) addresses the tamper-evidence requirement by binding records into a hash-linked chain replicated across mutually distrusting parties, and permissioned platforms such as Hyperledger Fabric (HLF) add an identity layer through a Membership Service Provider (MSP) and Public Key Infrastructure (PKI), making it feasible to say not merely that a record is unaltered but who produced it (Pajooh et al. 2022; Androulaki et al. 2018; Thakkar, Nathan, and Vishwanathan 2018).

Running that machinery at the edge is the difficulty. Consensus, endorsement and world-state maintenance were designed for datacentre hardware, whereas an agricultural deployment must place gateways in the field, on 64-bit Advanced RISC Machine architecture (ARM64) single-board computers, behind intermittent connectivity. Whether a permissioned ledger can meet irrigation-control timing requirements on such hardware has until recently been asserted rather than measured.

Finally, protection is not exhausted by integrity. A soil-moisture trace is also a disclosure. It reveals plot productivity, irrigation strategy and, in aggregate, the commercial position of the farmer. Where records are replicated across a consortium that includes buyers, insurers and cooperatives, the same replication that secures integrity enlarges the audience for private information. Identity, integrity and privacy therefore have to be treated as one design problem rather than three independent ones, which is the position this thesis takes.

## 1.2	Statement of the Problem

Three properties that ought to hold simultaneously are, in current systems, traded against one another.

Identity is weak at the sensing layer. Constrained nodes seldom carry credentials strong enough to authenticate individual readings. Where authentication exists it typically terminates at the gateway, so the ledger attests to what the gateway submitted rather than to what the sensor measured. The provenance chain has a gap exactly where falsification is cheapest, at the final hop between an individual node and the record that claims to represent it.

Integrity guarantees do not survive the move to the edge. Permissioned ledgers provide tamper evidence, but their published performance characterisations assume server-class hosts. On ARM64 gateways with constrained memory and intermittent links, it is not established that endorsement and ordering can complete inside the control loop of an irrigation decision, nor how the ledger behaves during network partition and recovery.

Privacy is sacrificed to obtain integrity. Replicating raw agronomic readings to every consortium member is the simplest way to make them verifiable and the worst way to keep them confidential. Existing deployments largely accept this trade, or push confidential data off chain in a way that reintroduces the trusted intermediary the ledger was meant to remove.

The three shortfalls, shown together in Figure 1.2, are not independent faults to be repaired one at a time. Each follows from the same scarcity, and a remedy applied to one is paid for out of the budget available to the others.

The three protection properties and the shortfall in each. Because all three draw on a single constrained budget, they form one design problem rather than three independent ones.

Underlying all three is a resource problem. The transmission budget that would pay for per-reading signatures, encryption and privacy-preserving encodings is already consumed by the raw payload itself. Any scheme that adds cryptographic protection without first reducing transmission cost will exhaust the node’s energy budget or the gateway’s channel capacity.

How, then, can identity, integrity and privacy be protected simultaneously in an agricultural data processing pipeline, end to end from constrained sensing node to distributed ledger, within the energy, memory and channel-capacity budgets that field deployment actually permits?

## 1.3	Rationale of the Study

The budget for protection has to come from somewhere. If CRT residue decomposition reduces per-packet airtime substantially, it frees channel capacity and node energy that can be reinvested in protection rather than merely banked as efficiency. The same decomposition has a second, less obvious property. An individual residue discloses little about the original value, so residues distributed across channels or custodians provide a natural substrate for confidentiality and for threshold reconstruction. A mechanism introduced for efficiency can thus be made to serve privacy, which is what makes the combination worth studying rather than treating transmission and security as separate layers.

The two mechanisms also compose cleanly. Residue decomposition and ledger verification operate on the same object at different points in its life, the first on the reading in flight and the second on the record at rest, and the quantity each is sensitive to is different: the radio cares about occupancy, the ledger about the size and count of committed transactions. A scheme that reduces occupancy without increasing transaction count therefore improves one without taxing the other, which is not generally true of protection mechanisms and is worth demonstrating rather than assuming.

There is also an evidential gap. Claims about edge blockchain feasibility are typically supported by simulation or short laboratory runs. A sustained field deployment on real hardware, through real weather and real connectivity failures, produces a different and more useful class of evidence, including the failure modes that simulation does not generate.

The applied case is direct. Smallholder and arid-region agriculture is where both the water-efficiency gains and the certification-fraud exposure are largest, and where infrastructure assumptions of existing systems are least likely to hold.

## 1.4	Objectives of the Study

### 1.4.1	Main Objective

To design, implement and evaluate a distributed blockchain-based framework that protects identity, integrity and privacy in agricultural data processing, and to validate it in a sustained field deployment on constrained edge hardware.

### 1.4.2	Specific Objectives

- Analyse the identity, integrity and privacy requirements of agricultural data pipelines, decomposing the threat surface from sensing node through gateway to ledger into an attacker model with stated capabilities and assumptions.

- Design an integrated protection scheme combining CRT multi-channel residue transmission, which lowers per-packet airtime while preserving exact reconstruction, with an identity layer that binds each reading to an authenticated originating node through HLF MSP credentials and Hierarchical Role-Based Access Control (HRBAC) policy enforced in chaincode.

- Implement the framework on a heterogeneous edge cluster of ESP32 sensing nodes and ARM64 Raspberry Pi (RPi) gateways running HLF under Raft ordering, deployed under real agricultural operating conditions.

- Evaluate the transmission, ledger and access-control layers against compression and centralised baselines on airtime, energy, throughput, latency and memory footprint, and assess the scalability and fault-tolerance envelope through queuing-theoretic analysis validated against measured load.

## 1.5	Research Questions

- To what extent can CRT residue decomposition reduce per-packet airtime and increase gateway packet capacity without loss of reconstruction fidelity?

- Can a permissioned distributed ledger sustain the throughput and latency required by real-time irrigation control when hosted entirely on ARM64 edge gateways?

- What identity binding is required for a ledger record to attest to the originating sensing node rather than merely to the submitting gateway, and what does that binding cost?

- What confidentiality is obtained by distributing residues across channels and custodians, and under what adversary model does it hold?

- How does the integrated framework behave as offered load approaches gateway and ledger saturation, and where are its practical limits?

## 1.6	Research Hypotheses

H1 (Transmission efficiency).

CRT residue transmission reduces per-packet airtime by at least 40% relative to raw transmission while maintaining at least 99% reconstruction accuracy.

H2 (Edge ledger performance).

HLF hosted on four RPi 4B gateways sustains a mean transaction latency below 2 s at an average throughput of at least 50.

H3 (Access-control overhead).

Enforcing Role-Based Access Control (RBAC) in chaincode preserves at least 50 and keeps mean smart-contract execution below 500 ms.

H4 (Field robustness).

Sustained field operation maintains at least 99% end-to-end reliability, with measured airtime within 5 ms of the analytical model.

H5 (Privacy preservation).

An adversary observing fewer than k of the k residue channels cannot reconstruct the original reading, and the residual information leakage remains below a stated bound.

## 1.7	Scope and Delimitations

In scope. Sub-GHz LoRa sensing networks of the order of fifty nodes; permissioned ledgers of the HLF family under Raft ordering; gateway hardware in the RPi 4B class; agronomic telemetry, specifically soil moisture, temperature, humidity and irrigation actuation; and the identity, integrity and privacy properties of that pipeline.

Out of scope. Public permissionless blockchains and their consensus economics; cellular and satellite backhaul; image-based and remote-sensing agronomy; agronomic model accuracy as such, which is treated as given rather than as an object of study; and formal cryptographic proofs of the underlying primitives, which are cited rather than re-derived.

Delimitations. Field validation is drawn from a single deployment site and a single cropping season, which bounds the claims that can be made about generalisation across soil types and climates. Node population is bounded by what could be instrumented. Behaviour beyond that population is established analytically and by synthetic load rather than by direct measurement.

## 1.8	Significance of the Study

For engineering practice, the work supplies measured rather than simulated evidence about what a permissioned ledger costs on edge hardware, and a transmission scheme that reduces that cost enough to make protection affordable.

For the research literature, it connects two bodies of work that have developed separately, namely residue arithmetic for communication efficiency and distributed ledgers for data provenance, and shows that the first can subsidise the second.

Methodologically, the work contributes a characterisation obtained under field conditions rather than in a laboratory. Failures that a testbed does not generate, mesh partitions, thermal derating, intermittent backhaul and the recovery behaviour that follows each, are exactly the ones that determine whether such a system is deployable, and they only appear when the system is left running.

For agricultural stakeholders, it offers a path to verifiable certification and insurance records that does not require farmers to surrender commercially sensitive detail to every consortium member.

## 1.9	Definition of Key Terms

Airtime

The time a radio occupies a channel to transmit one packet. It governs both node energy consumption and gateway capacity.

Residue

The remainder of a reading modulo one of a set of pairwise coprime moduli. A complete residue set uniquely determines the original value below the product of the moduli.

Integrity

The property that a record has not been altered since it was committed, and that alteration would be detectable.

Identity

The property that a record can be attributed to a specific authenticated principal, here a specific sensing node rather than merely a gateway.

Privacy

The property that a record’s content is disclosed only to principals authorised to see it, notwithstanding replication of the ledger.

Endorsement

In HLF, the process by which designated peers simulate a transaction and sign the result before ordering.

## 1.10	Organisation of the Thesis

Chapter One states the background, problem, rationale, objectives, questions and hypotheses. Chapter Two reviews the literature on precision agriculture and IoT sensing, transmission optimisation, distributed ledgers in agriculture, edge-resident consensus and privacy-preserving data processing, and identifies the gap this work addresses. Chapter Three presents the materials and methods: system architecture, CRT formulation, identity and access-control design, hardware implementation, deployment protocol and evaluation methodology. Chapter Four reports results and applications across the transmission, ledger, access-control and agricultural-outcome dimensions. Chapter Five discusses the findings against the hypotheses, states limitations and threats to validity, and sets out conclusions, recommendations and perspectives for further research.

# 2	Review of the Literature

## 2.1	Introduction

This chapter surveys the five bodies of work on which the thesis draws: precision agriculture and IoT sensing; low-power wide-area transmission and the airtime constraint that governs it; residue arithmetic and the CRT; distributed ledgers in agriculture and their migration to the network edge; and identity, access control and privacy for replicated records. Each is treated on its own terms, then criticised, and the chapter closes by naming what the five leave undone.

## 2.2	Precision Agriculture and IoT Sensing

### 2.2.1	From Wireless Sensor Networks to Managed Irrigation

Precision agriculture began as site-specific crop management, matching inputs to measured within-field variability rather than to field averages (Pierce and Nowak 1999; Zhang, Wang, and Wang 2002). Early instrumentation used wireless sensor networks for soil and microclimate monitoring (N. Wang, Zhang, and Wang 2006; Bogena et al. 2010), and the subsequent decade moved the field from measurement to actuation, with Long Range Wide Area Network (LoRaWAN) irrigation control among the clearest applications (Evans, Iversen, and Kim 2008). Reviews of the area agree on the technical progress and on a persistent weakness (Gebbers and Adamchuk 2010; Aydin and Hall 2021): integration between the sensing layer and the decision-support systems that consume it remains fragmented, so measurement quality is rarely the limiting factor on outcomes.

### 2.2.2	What the Sensing Literature Does Not Settle

Two questions are left open. First, almost all of this work treats the reading as trustworthy by construction: the sensor reports, the system believes it, and no mechanism attests that the value committed to storage is the value measured. Second, the economics that make a deployment viable on a smallholding, which force the node bill of materials to a few tens of dollars, are seldom carried through to the security design. A protection mechanism that assumes a secure element or a mains supply has assumed away the constraint that defines the problem.

## 2.3	Low-Power Wide-Area Transmission and the Airtime Constraint

### 2.3.1	The Airtime Model

LoRa is the dominant physical layer for agricultural telemetry, with demonstrated rural ranges of several kilometres and, in favourable terrain, up to 15 km (Adelantado et al. 2017; Li et al. 2020; Raj and Prasad 2017). Its cost is airtime. For a spreading factor SF, bandwidth BW and coding rate CR, the symbol duration and the time on air of a packet carrying PL payload bytes are

Tsym=2SFBW,

Tair=Tpreamble+Tsym​8+max​8PL−4SF+28+164SF−2DECR+4, 0,

where DE is the low-data-rate optimisation flag. The term that matters is the ceiling: airtime grows in discrete steps with payload length, and every additional symbol is charged at Tsym, which doubles with each increment of SF (Adelantado et al. 2017; Bor et al. 2016).

Airtime is not merely an energy cost. Sub-GHz bands impose a duty-cycle limit, so a node that has transmitted for Tair must remain silent for a proportional interval, which caps reporting frequency independently of battery capacity. Under the unslotted access that LoRaWAN class A uses, the offered load per channel over a reporting window Tw for N nodes is

G=N⋅TairTw,

and the pure-ALOHA collision probability follows as

Pcoll=1−e−2G≈2G for small G,

which is linear in Tair in the low-load regime and degrades sharply as G approaches unity (Bor et al. 2016; Adelantado et al. 2017). Reducing per-packet airtime therefore buys capacity twice over, once through the duty cycle and once through the collision term.

### 2.3.2	Compression and Sampling Responses

The conventional response is to shorten the payload. Lossless coders adapted to sensor streams, principally LZW and Huffman variants, achieve useful ratios on agronomic data (Li, Da Xu, and Zhao 2018; Marcelloni and Vecchio 2008), but their dictionary and buffer requirements are measured in kilobytes, which is a significant fraction of the free RAM on a microcontroller already running a radio stack and a sensor driver (Marcelloni and Vecchio 2010). Adaptive sampling transmits only when a reading departs from a predicted trajectory (Kjellsson and Zakhor 2008), at the risk of suppressing exactly the transient that mattered. Aggregation and protocol-level optimisation (Heinzelman, Chandrakasan, and Balakrishnan 2000; Sanchez et al. 2021) reduce traffic at the cost of resolution or latency.

### 2.3.3	The Common Limitation

Every one of these approaches reduces the number of bytes sent. None reduces the occupancy of an individual packet while preserving the reading exactly, and occupancy is what Equations [eq:offered_load] and [eq:collision] are sensitive to. This distinction, between total volume and per-packet occupancy, is where the present work departs from the compression literature.

## 2.4	Residue Arithmetic and the Chinese Remainder Theorem

### 2.4.1	Statement and Reconstruction

Let m1,m2,…,mk be pairwise coprime moduli and M=i=1kmi. For any integer x with 0≤x<M, the residue vector

r=r1,r2,…,rk,  ri=x mod mi,

determines x uniquely, and x is recovered by

x=i=1kriMiNi mod M,  Mi=Mmi, Ni=Mi−1 mod mi.

Reconstruction is exact because MiNi≡1 (mod mi) and Mi≡0 (mod mj) for j≠i (Hardy et al. 2008; Ding, Pei, and Salomaa 1996). Evaluated by Garner’s algorithm the cost is Ok modular operations with no division in the inner loop (Garner 1959), which is what makes the scheme viable on a gateway rather than only on a workstation. Figure 2.1 shows the decomposition and its inverse.

Residue decomposition and reconstruction. One long packet is replaced by k short ones that a multi-channel concentrator can demodulate simultaneously, and the original value is recovered exactly in Ok operations.

### 2.4.2	Where Residue Systems Have Been Applied

Residue number systems built on Equation [eq:crt_reconstruction] are long established in computer arithmetic, where carry-free addition and multiplication across residue channels accelerate arithmetic and support fault-tolerant computation (Parhami 2010; Shoup 2009). The theorem is equally standard in cryptography (Ding, Pei, and Salomaa 1996; Garner 1959) and in error correction (Goldreich, Ron, and Sudan 1999). Within sensor networks, Y. Wang, Chen, and Wang (2018) applied residue representations to reduce stored data volume.

### 2.4.3	The Gap in the Residue Literature

The sensor-network application targeted storage, not transmission, and therefore did not exploit the property that matters here: that k residues are k independent short packets which a multi-channel concentrator can receive at the same instant. The computer-arithmetic literature exploits parallelism, but across arithmetic units rather than across radio channels, where the currency is occupancy rather than processor cycles. No prior system, to the author’s knowledge, uses residue decomposition to drive parallel demodulation on a low-power radio.

## 2.5	Distributed Ledgers in Agriculture

### 2.5.1	Supply-Chain Transparency

The earliest and best-developed application is provenance along the food supply chain. Tian (2016) combined RFID with a blockchain to trace agri-food products end to end, and subsequent surveys map the field (Kamilaris, Fonts, and Prenafeta-Boldó 2019; Galvez, Mejuto, and Simal-Gandara 2018) and its food-safety consequences (Bosona and Gebresenbet 2023). The common architecture places the ledger between organisations that have commercial reason to distrust one another, which is precisely the setting in which replicated tamper-evident storage earns its cost.

### 2.5.2	Farm Data Management and Smart-Contract Automation

A second line applies the ledger inside the farm, to data ownership and access rather than to product movement (Lin et al. 2018; Ferrag et al. 2020; Haque, Bhushan, and Dhiman 2024). A third automates agreements, using smart contracts to trigger payments, insurance settlement or irrigation actions (Rama et al. 2021; Patil et al. 2022). Both assume a ledger that is reachable and responsive.

### 2.5.3	The Recurring Weakness

Across all three lines, implementations that operate under the constraints of real-time control on resource-constrained hardware remain scarce. Where performance is reported it is typically obtained on server-class hosts, and the question of whether endorsement and ordering complete inside an irrigation control loop is not usually asked.

## 2.6	Edge-Resident Consensus and Constrained Hardware

### 2.6.1	Edge Computing in Agricultural Systems

Moving computation to the field reduces round-trip latency and reduces dependence on backhaul that rural sites often lack (Sharma and Wang 2020). Liu et al. (2019) reported improved response times for irrigation control under edge processing, and X. Wang et al. (2021) showed that keeping data local improves privacy by limiting what leaves the site. ARM64 single-board clusters are now routinely used for distributed sensing and inference (Sharma and Wang 2020; X. Wang et al. 2021).

### 2.6.2	Networking the Edge

Agricultural deployments overwhelmingly use star topologies, in which a single gateway failure removes a zone. Mesh routing has been studied mostly in urban and industrial settings (Akyildiz and Wang 2005); BATMAN-adv has shown promise for rural connectivity (Schmidt et al. 2010) without being taken up in precision agriculture. Hybrid multi-technology architectures have been proposed (Chettri and Bera 2020) but tend to add management complexity without integrating cleanly.

### 2.6.3	Capacity Limits

Where a gateway serves C channels over a window W and each node consumes Teff of effective airtime per reporting cycle, the node ceiling is

Nmax=C⋅WTeff,

and per-channel packet throughput, for a concentrator that demodulates one packet per channel concurrently, is

λ=1Tair.

Equations [eq:node_ceiling] and [eq:packet_rate] pull in different directions once a reading is split across channels: splitting raises the total airtime a node consumes, and so lowers Nmax, while lowering Tair per packet, and so raising λ. Which of the two binds depends on whether the deployment is limited by node count or by packet capacity, a distinction the existing literature does not draw.

### 2.6.4	What Is Missing at the Edge

Edge implementations rarely integrate with verifiable storage, so the trust gap that motivates the ledger reappears the moment computation is moved off the ledger. Communication parallelism, meaning simultaneous use of multiple wireless channels by one logical reading, is essentially unexplored.

## 2.7	Identity, Access Control and Privacy

### 2.7.1	Identity and Access Control

Permissioned ledgers supply identity through a MSP backed by a PKI, which allows a record to be attributed to an enrolled principal rather than merely to an address (Pajooh et al. 2022; Androulaki et al. 2018). RBAC and its hierarchical variants are the usual policy layer, enforced in chaincode so that the authorisation decision is itself auditable (Ferrag et al. 2020).

### 2.7.2	Privacy Under Replication

Replication is what makes a ledger tamper-evident and is also what makes it leak. Where a consortium includes buyers, insurers and cooperatives, every member sees every replicated record, and agronomic traces disclose plot productivity and irrigation strategy. Keeping data local reduces the exposure but reintroduces the trusted party the ledger was meant to remove (X. Wang et al. 2021).

### 2.7.3	The Unresolved Tension

The literature offers identity and integrity together, and privacy separately, usually by withholding data from the ledger. What it does not offer is a mechanism in which the same operation that makes transmission affordable also constrains disclosure. That possibility, latent in the residue representation of Equation [eq:residues], is the opening this thesis works in.

## 2.8	Research Gaps

Table 2.1 states the gaps that follow from the five preceding sections and the objective that addresses each.

Gaps identified in the literature and the objective addressing each.

| Area | Gap | Objective |
| --- | --- | --- |
| Transmission cost | Compression reduces total volume, not per-packet occupancy, and costs memory a constrained node does not have | O2 |
| Residue arithmetic | Applied to storage and to arithmetic units, never to parallel demodulation on a low-power radio | O2 |
| Edge ledgers | Performance characterised on server hardware; behaviour on ARM64 gateways under partition unmeasured | O3, O4 |
| Identity | Authentication terminates at the gateway, so the record attests to the submitter rather than the sensor | O1, O2 |
| Privacy | Confidentiality obtained by withholding data, which reinstates a trusted intermediary | O1, O2 |
| Capacity | No validated model distinguishing node-count from packet-capacity limits for multi-channel residue traffic | O4 |

## 2.9	Partial Conclusion

The sensing literature establishes what to measure and treats the measurement as trustworthy. The transmission literature establishes that airtime binds and attacks it by shortening payloads, which does not address per-packet occupancy. Residue arithmetic supplies a decomposition whose parallelism has been exploited in silicon but not on the air. The ledger literature supplies tamper evidence and identity, characterised on hardware that agricultural deployments do not have. The privacy literature obtains confidentiality by withholding, which forfeits verifiability.

The four therefore leave a single composite question, which the remainder of this thesis addresses: whether a decomposition adopted to reduce per-packet occupancy can simultaneously carry node-level identity and bound disclosure, and whether the resulting system holds up on the hardware and in the conditions that field deployment actually imposes.

## REFERENCES

# Supplementary Material

Abioye, Emmanuel Abiodun, Mohammad Shukri Zainal Abidin, Mohd Saiful Azimi Mahmud, Salinda Buyamin, Mohamad Hafis Izran Ishak, Muhammad Khairie Idham Abd Rahman, Abdulrahaman Okino Otuoze, Patrick Onotu, and Muhammad Shahrul Azwan Ramli. 2020. “A Review on Monitoring and Advanced Control Strategies for Precision Irrigation.” Computers and Electronics in Agriculture 173: 105441. https://doi.org/10.1016/j.compag.2020.105441.

Adelantado, Ferran, Xavier Vilajosana, Pere Tuset-Peiro, Borja Martinez, Joan Melia-Segui, and Thomas Watteyne. 2017. “Understanding the Limits of LoRaWAN.” IEEE Communications Magazine 55 (9): 34–40. https://doi.org/10.1109/MCOM.2017.1600613.

Akyildiz, Ian F., and Xudong Wang. 2005. “A Survey on Wireless Mesh Networks.” IEEE Communications Magazine 43 (9): S23–30. https://doi.org/10.1109/MCOM.2005.1509968.

Androulaki, Elli, Artem Barger, Vita Bortnikov, Christian Cachin, Konstantinos Christidis, Angelo De Caro, David Enyeart, et al. 2018. “Hyperledger Fabric: A Distributed Operating System for Permissioned Blockchains.” In Proceedings of the 13th EuroSys Conference, 30:1–15. https://doi.org/10.1145/3190508.3190538.

Aydin, Gokhan, and Michael J. Hall. 2021. “A Comprehensive Review of IoT and Wireless Sensor Networks in Precision Agriculture.” IEEE Access 9: 157733–55. https://doi.org/10.1109/ACCESS.2021.3129552.

Bogena, Heye R., Johan A. Huisman, Christian Oberdörster, and Harry Vereecken. 2010. “Evaluation of a Low-Cost Soil Water Content Sensor for Wireless Network Applications.” Journal of Hydrology 344 (1): 32–42. https://doi.org/10.1016/j.jhydrol.2007.06.032.

Bor, Martin C., Utz Roedig, Thiemo Voigt, and Juan M. Alonso. 2016. “Do LoRa Low-Power Wide-Area Networks Scale?” In Proceedings of the 19th ACM International Conference on Modeling, Analysis and Simulation of Wireless and Mobile Systems (MSWiM), 59–67. ACM. https://doi.org/10.1145/2988287.2989163.

Bosona, Techane, and Girma Gebresenbet. 2023. “The Role of Blockchain Technology in Promoting Traceability Systems in Agri-Food Production and Supply Chains.” Sensors 23 (11): 5342. https://doi.org/10.3390/s23115342.

Chettri, Lalit, and Rabindra Bera. 2020. “A Comprehensive Survey on Internet of Things (IoT) Toward 5G Wireless Systems.” IEEE Internet of Things Journal 7 (1): 16–32. https://doi.org/10.1109/JIOT.2019.2948888.

Ding, Cunsheng, Dingyi Pei, and Arto Salomaa. 1996. Chinese Remainder Theorem: Applications in Computing, Coding, Cryptography. World Scientific Publishing. https://doi.org/10.1142/2893.

Dono, Gabriele, Luca Giraldo, and Simone Severini. 2010. “The Irrigation of Agricultural Land in Italy: Main Economic Implications and Effects in an Ongoing Climate Change.” Agricultural Water Management 97 (7): 949–57. https://doi.org/10.1016/j.agwat.2010.01.025.

Evans, R. G., W. M. Iversen, and Y. Kim. 2008. “Integration of Wireless Sensor Network with Variable Rate Irrigation Technology.” Proceedings of the ASABE Annual International Meeting 12: 1–13.

Ferrag, Mohamed Amine, Lei Shu, Xianjun Yang, Abdelouahid Derhab, and Leandros Maglaras. 2020. “Security and Privacy for Green IoT-Based Agriculture: Review, Blockchain Solutions, and Challenges.” IEEE Access 8: 32031–53. https://doi.org/10.1109/ACCESS.2020.2973178.

Galvez, Juan F., Javier Mejuto, and Jesus Simal-Gandara. 2018. “Future Challenges on the Use of Blockchain for Food Traceability Analysis.” TrAC Trends in Analytical Chemistry 107: 222–32. https://doi.org/10.1016/j.trac.2018.08.011.

Garner, Harvey L. 1959. “The Residue Number System.” IRE Transactions on Electronic Computers EC-8 (2): 140–47. https://doi.org/10.1109/TEC.1959.5219515.

Gebbers, Robin, and Viacheslav I. Adamchuk. 2010. “Precision Agriculture and Food Security.” Science 327 (5967): 828–31. https://doi.org/10.1126/science.1183899.

Goldreich, Oded, Dana Ron, and Madhu Sudan. 1999. “Chinese Remaindering with Errors.” In Proceedings of the 31st Annual ACM Symposium on Theory of Computing, 225–34. https://doi.org/10.1145/301250.301308.

Haque, Abu Bakar, Bharat Bhushan, and Gaurav Dhiman. 2024. “Conceptualizing Smart City Applications: Requirements, Architecture, Security Issues, and Emerging Trends.” Expert Systems 41 (5): e13382. https://doi.org/10.1111/exsy.13382.

Hardy, Godfrey Harold, Edward Maitland Wright, Roger Heath-Brown, and Joseph Silverman. 2008. An Introduction to the Theory of Numbers. 6th ed. Oxford University Press.

Haxhibeqiri, Jetmir, Eli De Poorter, Ingrid Moerman, and Jeroen Hoebeke. 2018. “A Survey of LoRaWAN for IoT: From Technology to Application.” Sensors 18 (11): 3995. https://doi.org/10.3390/s18113995.

Heinzelman, Wendi Rabiner, Anantha Chandrakasan, and Hari Balakrishnan. 2000. “Energy-Efficient Communication Protocol for Wireless Microsensor Networks.” In Proceedings of the 33rd Annual Hawaii International Conference on System Sciences, 10–20. https://doi.org/10.1109/HICSS.2000.926982.

Kamilaris, Andreas, Agusti Fonts, and Francesc X. Prenafeta-Boldó. 2019. “The Rise of Blockchain Technology in Agriculture and Food Supply Chains.” Trends in Food Science & Technology 91: 640–52. https://doi.org/10.1016/j.tifs.2019.07.034.

Kjellsson, Jesper, and Avideh Zakhor. 2008. “Adaptive Sampling for Sensor Networks.” ACM Transactions on Sensor Networks 4 (3): 1–39. https://doi.org/10.1145/1362542.1362545.

Li, Shancang, Li Da Xu, and Shanshan Zhao. 2018. “5G Internet of Things: A Survey.” Journal of Industrial Information Integration 10: 1–9. https://doi.org/10.1016/j.jii.2018.01.005.

Li, Shancang, Jie Li, Jia Liu, Hao Wang, and Shanshan Zhao. 2020. “Performance Analysis of LoRa Networks for Smart Agriculture.” IEEE Internet of Things Journal 7 (10): 10045–57. https://doi.org/10.1109/JIOT.2020.2999323.

Lin, Jian, Zhiqi Shen, Anting Zhang, and Yi Chai. 2018. “Blockchain and IoT Based Food Traceability System for Smart Agriculture.” In Proceedings of the 3rd International Conference on Crowd Science and Engineering, 1–6. https://doi.org/10.1145/3265689.3265692.

Liu, Yang, Cheng Yang, Li Jiang, Shengli Xie, and Yan Zhang. 2019. “Intelligent Edge Computing for IoT-Based Energy Management in Smart Cities.” IEEE Network 33 (2): 111–17. https://doi.org/10.1109/MNET.2019.1800254.

Marcelloni, Francesco, and Massimo Vecchio. 2008. “A Simple Algorithm for Data Compression in Wireless Sensor Networks.” IEEE Communications Letters 12 (6): 411–13. https://doi.org/10.1109/LCOMM.2008.080300.

———. 2010. “An Efficient Lossless Compression Algorithm for Periodic Data in Wireless Sensor Networks.” The Computer Journal 53 (6): 822–33. https://doi.org/10.1093/comjnl/bxp118.

Pajooh, Houshyar Honar, Mohammad A. Rashid, Fakhrul Alam, and Serge Demidenko. 2022. “Experimental Performance Analysis of a Scalable Distributed Hyperledger Fabric for a Large-Scale IoT Testbed.” Sensors 22 (13): 4868. https://doi.org/10.3390/s22134868.

Parhami, Behrooz. 2010. Computer Arithmetic: Algorithms and Hardware Designs. 2nd ed. Oxford University Press.

Patil, Akshay S., Bayu Adhi Tama, YoungHo Park, and Kyung-Hyune Rhee. 2022. “A Framework for Blockchain-Based Smart Farming System Using IoT.” IEEE Access 10: 6845–62. https://doi.org/10.1109/ACCESS.2022.3142563.

Pereira, Luis Santos, Theib Oweis, and Akissa Zairi. 2002. “Irrigation Management Under Water Scarcity.” Agricultural Water Management 57 (3): 175–206. https://doi.org/10.1016/S0378-3774(02)00075-6.

Pierce, Francis J., and Peter Nowak. 1999. “Aspects of Precision Agriculture.” Advances in Agronomy 67: 1–85. https://doi.org/10.1016/S0065-2113(08)60513-1.

Raj, Emilin Renu, and D. V. Prasad. 2017. “A Comprehensive Study on LoRa Technology for Wireless Sensor Networks.” International Journal of Advanced Research in Computer Science 8 (5): 1214–19.

Rama, R., V. Kanchana, N. Sivasankari, and S. Prabha. 2021. “A Systematic Review of Blockchain Technology for Agriculture Supply Chain Management.” Materials Today: Proceedings 47: 3148–54. https://doi.org/10.1016/j.matpr.2021.06.253.

Rathi, M., and C. Gomathy. 2025. “Smart Agriculture Resource Allocation and Energy Optimization Using Bidirectional Long Short-Term Memory with Ant Colony Optimization (Bi-LSTM–ACO).” Frontiers in Communications and Networks 6: 1587402. https://doi.org/10.3389/frcmn.2025.1587402.

Sanchez, Sergio, Sylvie Blanc, Pedro Yuste, and Juan J. Serrano. 2021. “Energy-Efficient Data Transmission in Wireless Sensor Networks for Precision Agriculture.” Sensors 21 (4): 1209. https://doi.org/10.3390/s21041209.

Schmidt, A., M. Westenberger, J. Seitz, K. Kuladinithi, and A. Timm-Giel. 2010. “B.a.t.m.a.n. For Mobile Ad-Hoc Networks.” Proceedings of IEEE CCNC, 1–6. https://doi.org/10.1109/CCNC.2010.5421784.

Sharma, Sandeep K., and Xianbin Wang. 2020. “Live Data Analytics with Collaborative Edge and Cloud Processing in Wireless IoT Networks.” IEEE Internet of Things Journal 7 (7): 6071–84. https://doi.org/10.1109/JIOT.2020.2977767.

Shoup, Victor. 2009. A Computational Introduction to Number Theory and Algebra. 2nd ed. Cambridge University Press.

Thakkar, Parth, Senthil Nathan, and Balaji Vishwanathan. 2018. “Performance Benchmarking and Optimizing Hyperledger Fabric Blockchain Platform.” In 2018 IEEE 26th International Symposium on Modeling, Analysis, and Simulation of Computer and Telecommunication Systems (MASCOTS), 264–76. https://doi.org/10.1109/MASCOTS.2018.00034.

Tian, F. 2016. “An Agri-Food Supply Chain Traceability System for China Based on RFID and Blockchain Technology.” 2016 13th International Conference on Service Systems and Service Management, 1–6. https://doi.org/10.1109/ICSSSM.2016.7538424.

Vukovic, Vukica, Darijo Raca, and Milan Vukovic. 2023. “An Advanced Energy-Efficient Environmental Monitoring in Precision Agriculture Using LoRa-Based Wireless Sensor Networks.” Sensors 23 (14): 6332. https://doi.org/10.3390/s23146332.

Wang, Ning, Naiqian Zhang, and Maohua Wang. 2006. “Wireless Sensors in Agriculture and Food Industry: Recent Development and Future Perspective.” Computers and Electronics in Agriculture 50 (1): 1–14. https://doi.org/10.1016/j.compag.2005.09.003.

Wang, Xiaofei, Yi Han, Victor C. M. Leung, Dusit Niyato, Xueqiang Yan, and Xu Chen. 2021. “Convergence of Edge Computing and Deep Learning: A Comprehensive Survey.” IEEE Communications Surveys & Tutorials 22 (2): 869–904. https://doi.org/10.1109/COMST.2020.2970550.

Wang, Yong, Irene Chen, and Dazhi Wang. 2018. “A Novel Data Compression Method for Wireless Sensor Networks.” Sensors 18 (4): 1057. https://doi.org/10.3390/s18041057.

Zhang, Naiqian, Maohua Wang, and Ning Wang. 2002. “Precision Agriculture: A Worldwide Overview.” Computers and Electronics in Agriculture 36 (2): 113–32. https://doi.org/10.1016/S0168-1699(02)00096-0.